Job:

# RP2100™

# ASME Series

# Atmospheric Pool Heaters

# Models 185B-405B

Engineer:

Contractor:

Prepared By: 	     		Date:

Model: 	     		Indoor/Outdoor

- 82% Efficiency

- ASME Code Glass-Lined Cast Iron Headers

- ASME Code Copper Fin Tube

- 125 lb. Pressure Relief Valve

- Small Footprint

- Unitherm Governor Anti-Condensation Control

- Internal Bypass for High Flow Rates (up to 125 gpm)

Indoor/Outdoor

Digital model with outdoor top shown

#### Heat Exchanger

- ASME Inspected and Stamped
160 PSIG

- National Board Approved

- Glass-lined Cast Iron Headers

- Finned Tubing

Copper – Standard

Cupro Nickel – Optional

- ASME Steel Tube Sheet

- Silicone O-Rings

- 125 PSIG Pressure Relief Valve

- Water Connections

Right Hand – Standard

Left Hand – Field Change

- Flow Configuration – Two-pass

#### Controls – Millivolt Option

- Standing Pilot Ignition

- Thermopile Generator

- Single Mechanical Sensor

- Flame Rollout Switch

- High Limit Control

- On/Off Switch

- Water Pressure Switch

#### Controls – Digital Option

- 115/230V, 60Hz, 1 Ph Power Supply

- 115/230/24V Transformer

- 100% Pilot Shut-off; Lockout – Propane units only

- Electronic, Intermittent Ignition Pilot (IID)

- High Limit Control

- On/Off Switch

- Flame Rollout Switch

- Water Pressure Switch

- Dual Electronic Temp Sensor

- Digital Display With Diagnostics

#### Gas Control Train

- Fuel

- Natural Gas

- Propane

- Combination Control Valve

- On/Off Firing

- ANSI Z21.56 Design Certified

#### Construction

- Front Controls

- Stainless Steel Burners

- Polytuf Powder Coat Finish

- Indoor/Outdoor

- Vent Selection

Outdoor Stackless Top – Standard

Indoor Draft Diverter– Loose

D-2 Power Vent – Loose

- Base (Optional)

Combustible Floor Shield

RP2100 – ASME Series

Raypak, Inc.  2151 Eastman Avenue, Oxnard, CA  93030  (805) 278-5300  Fax (800) 872-9725   www.raypak.com

Raypak Canada Ltd.  2805 Slough St., Mississauga, Ontario, Canada L4T 1G2  (905) 677-7999  Fax (905) 677-8036  www.raypakcanada.com

Atmospheric Pool Heaters	Model

| Heater Model | BTUH Input (000) | (A) Cabinet Width | (B) Flue Dia. | (C) Indoor Drafthood | (J) | Gas Conn. | Water Conn. | Shipping Weights (lbs) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Heater with Stackless Top | Indoor Drafthood |
| R185B | 181 | 18-1/4” | 6” | 62-5/8” | 12-1/16” | 3/4" | 2” | 191 | 12 |
| R265B | 264 | 22-3/8” | 7” | 62-7/8” | 11-1/8” | 3/4" | 2” | 214 | 15 |
| R335B | 334 | 25-3/4” | 8” | 63-3/4” | 10-3/4” | 3/4" | 2” | 234 | 17 |
| R405B | 399 | 29-1/4” | 9” | 65-3/8” | 12-1/2” | 3/4" | 2” | 253 | 20 |

NOTES:

Model number suffix indicates pilot type (E = digital, M = millivolt) and fuel type (P = propane, N = natural gas). Prefix “C” indicates Cast Iron (ASME) headers. Example: Model number CR-405B-EN indicates an ASME unit with digital (IID) ignition using natural gas.

Ratings are for natural gas and elevations up to 2,000 feet above sea level. Reduce input 4% for each 1,000 feet above sea level when installed at over 2,000 feet elevation.

Manufactured under Patent  No. 3,623,458.