Job:

# Instant-Pak® Commercial Tankless

## Water Heaters

## Models NH-188 & NH-208

Engineer:

Contractor:

Prepared By: 	     		Date:

Model:

High Efficiency

Stepless modulating, 16:1 Near condensing design (.82 Energy Factor) meets or exceeds Energy Star Standards.

Low Emissions

Low NOx design meets latest Green Legislation for 2012 in North America.

Commercial Duty

Instant-Pak can generate 185ºF (85 ºC) temp hot water on

demand without modification.

Electronic Ignition

Direct Spark Ignition

Compact, Lightweight Design

Mounts easily onto walls without heavy bracing.

Easy to Plumb, Easy to Service

Concentric Vent connection on top for indoor unit. Inlet and outlet water piping, gas connections and wiring at bottom. Remote Control (included) mounts easily for convenient access.

Safe Operation

Sealed combustion chamber runs cool and sensors shut down the tankless heater in the event of a fault.

Electronic Thermostat with Diagnostics

Electronic solid state thermostatic control operates the heater in the most energy efficient manner. Faults are displayed on the supplied Remote’s three digit display and specific LEDs.

.

#### Heat Exchanger

- Finned Copper Tube

- Nine Pass Heat Exchanger

- All Non-Ferrous Waterways

Energy Factor

| Model | E F |
| --- | --- |
| NH-188 | .82 |
| NH-208 | .82 |

#### Controls

- 120V, 60Hz, 1 Power Supply

- Electronic, Direct  Spark Ignition (DSI)

- 100% Flame Monitor Shut-off/Lockout

- High Limit Control

- Operating Limit Control

- Guardian OFWTM Heat Exchanger Overtemp Breach Protection

- Water Pressure Based Activation Switch

- Electronic Flame Monitoring Based Blocked Flue Sensor

#### Venting Options

- UL 1738 Approved Stainless Steel Vent

- 3” x 5” ( 80 x 125mm) Concentric Vent

- Horizontal or Vertical Vent Termination

- Gas Control Train

- Natural Gas or Propane

- 16:1 Modulation

- Manual Gas Shut-Off Valve

- Redundant Valve Feature

#### Construction

- Indoor and Outdoor Construction

- Remote Control Included

- 120 VAC, Pre-wired Power Cord

- Appliance Grade Finish

Raypak Instant-Pak Commercial

Near Condensing Tankless Water Heater	Model

Indoor Models: NH-188DV and NH-208DV                               Outdoor Models; NH-188X and NH-208X

| Model | Input MBTUH (Kw) | Piping Connections |  | Concentric Vent Diameter | Approx. Shipping Weight lbs (Kg) | Clearances from Combustibles |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Water NPT (mm) | Gas NPT (mm) |  |  | Front (mm) | Other Sides (mm) | Top (mm) | Flue Vent (mm) |
| NH-188 (X & DV) | 180 (52.7) | 3/4" (N/A) | 3/4" (N/A) | 3" X 5" 80mm X 125mm (DV Only) | 54 (24.5) | 12” (300) | 1" (25) | 12” (300) | 0" (0) |
| NH-208 (X & DV) | 199 (58.3) | 3/4" (N/A) | 3/4" (N/A) | 3" X 5" 80mm X 125mm (DV Only) | 54 (24.5) | 12” (300) | 1" (25) | 12” (300) | 0" (0) |

Raypak, Inc. 2151 Eastman Avenue, Oxnard, CA  93030 (805) 278-5300 Fax (800) 872-9725   www.raypak.com