Job:

# Raytherm™ Residentials

## Hydronic Heating Boilers

## Models 42-180

Engineer:

Contractor:

Prepared By: 	     		Date:

Model:

Applications

- Radiant Heating

- Hydronic Space Heating

- Snow Melting

High Efficiency

Heating efficiencies meet or exceed NAECA standards.

Factory Installed Circulator and Bypass

Thermostatically-controlled circulator maintains proper flow through the boiler. As a packaged system, installation is fast and easy.

Electronic Ignition

Intermittent pilot ignition improves system efficiency by using fuel only when there is a call for heat.

Compact, Lightweight Design

Fig. #8942

Even the largest model slips easily into tight spaces.

Easy to Plumb, Easy to Service

Inlet and outlet water piping, gas connection, wiring and controls are all up front for easy access.

Safe Operation

Built-in manual reset safety sensors shut down the boiler in the event of a flame roll-out or flue gas spillage. Normal working pressure is 125 PSI max.

Fast Response Temperature Sensor

Electronic solid state auto-reset control shuts down the boiler in the event of over-heating caused by a low-flow condition.

#### Heat Exchanger

- ASME Inspected and Stamped
160 PSIG

- National Board Approved

- Glass-lined Cast Iron Headers

- Copper Finned Tubing

- ASME Steel Tube Sheet

- Silicone O-Rings

- 30 PSIG ASME Pressure Relief Valve

- Temperature and Pressure Gauge

- Cast Iron Pump – 120 VAC, Mounted

- Bypass Assembly

#### Stainless Options

- All Stainless Pump, 120 VAC, Mounted Stainless Fittings

AFUE Ratings

| Model | AFUE |
| --- | --- |
| 0042 | 84.0 |
| 0066 | 83.0 |
| 0090 | 82.7 |
| 0135 | 82.8 |
| 0180 | 83.2 |

#### Controls

- 120V, 60Hz, 1 Ø Power Supply

- 120/24V Transformer

- Electronic, Intermittent Ignition
(IID) - Standard

- 100% Pilot Shut-off/Lockout (IID only)

- High Limit Control

- Operating Limit Control

- Flow Sensor

- Flue Gas Spillage Sensor

- Flame Rollout Sensor

- Automatic Vent Damper

- Electronic Outdoor Reset Control

#### Gas Control Train

- Fuel

Natural Gas

Propane Gas

- Firing Mode

- On/Off - Models 42-66

- Two-stage Firing - Models 90-180

- Combination Gas Valve

- Manual Shut-off

- Redundant Valve Features

- Gas Pressure Regulator

#### Construction

- Indoor Construction

- Enclosed Front Controls

- Stainless Steel Burners

- Polytuf Powder Coat Finish

- Built-in Draft Diverter

- Certified for combustible flooring

Raypak Residentials

Hydronic Heating Boilers	Model

Models 42, 66 & 90

Models 135 & 180

Raypak, Inc.  2151 Eastman Avenue, Oxnard, CA  93030  (805) 278-5300  Fax (800) 872-9725  www.raypak.com

Notes:

1.	Standard models, use “H-” prefix (e.g., H-0042); Non-ferrous models, use “HB-” prefix (e.g., HB-0042)

2.	Per Section 11.1.2 of the National Fuel Gas Code, derate the appliance firing rate 4% per 1,000 feet above sea level for installations above 2,000 feet.