Job:

# Condensing 90+

# XPak™ Residential

## Hydronic Heating Boilers

## Models 85 &120

Engineer:

Contractor:

Prepared By: 	     		Date:

Model:

Applications

- Radiant Heating

- Hydronic Space Heating

- Snow Melting

- DHW via indirect

High Efficiency

Infinitely modulating, Condensing design meets or exceeds Energy Star Standards.

Low Emissions

Low NOx design meets all current Green Legislation in North America.

Factory Installed Circulator

w/ Bypass and Condensate Siphon

Thermostatically-controlled circulator maintains proper flow through the boiler. As a packaged system, installation is fast and easy. Twin chamber ball type

Siphon, the best flue seal.

Electronic Ignition

Direct Spark Ignition

Compact, Lightweight Design

Mounts easily onto walls without heavy bracing.

Easy to Plumb, Easy to Service

Vent on top. Inlet and outlet water piping, gas connections, wiring and condensate drain at bottom. Controls are up front for easy access.

Safe Operation

Sealed combustion chamber runs cool and sensors shut down the boiler in the event of a fault.

Electronic Thermostat with Diagnostics

Electronic solid state thermostatic control operates the boiler in the most efficient manner. Faults are displayed on two digit display.

Whisper Quiet Operation

Less than 60 dba

DHW Temp Controller Up Front

#### Heat Exchanger

- ASME Inspected and Stamped
45 PSIG

- National Board Approved

- Extruded 6063-T1 Aluminum Alloy Single Pass Heat Exchanger

- 45 PSIG ASME Pressure Relief Valve

- Pressure Gauge

- 3 Speed Non-Ferrous Pump

- 120 VAC, Mounted

- Two Automatic Air Vents

AFUE Ratings

| Model | AFUE |
| --- | --- |
| 85 | 90.3 |
| 120 | 90.5 |

#### Controls

- 120V, 60Hz, 1Power Supply

- 120/24V Transformer

- Electronic, Direct  Spark Ignition (DSI)

- 100% Flame Monitor Shut-off/Lockout

- High Limit Control

- Operating Limit Control

- Pressure Switch

- Condensate High Level Sensor

- Flue Sensor

- Flue Limit

#### Venting Options

- AL29-4C Stainless Steel Vent

- CPVC Plastic Vent

- Polypropylene Plastic Vent

- Concentric Vent or Twin Penetrations

- Horizontal or Vertical Vent

- Gas Control Train

- Natural Gas (Propane Conversion

- Kit w/ Every Unit)

- 4 to 1 Modulation

- Zero Governor Gas Valve

- Manual Gas Shut-Off Valve

- Redundant Valve Feature

- Negative Pressure Gas Regulator

#### Construction

- Indoor Construction

- Enclosed Front Controls

- Cylindrical Stainless Steel Burner

- Appliance Grade Finish

Raypak XPak Residential

Hydronic Heating Boilers	Model

Models 85 &120

Note:

1.	    DHW capability provided via following options:  three way valve or pump and indirect tank

Raypak, Inc. 2151 Eastman Avenue, Oxnard, CA  93030 (805) 278-5300 Fax (800) 872-9725   www.raypak.com